Tech Specifications:

Reolink 4K TrackMix 4G Solar Battery Auto PTZ AI Camera + 64GB SD

- Image Sensor: 1/3″ + 1/2.7″ CMOS Sensor

- Video Resolution: Up to 3840 × 2160 (8MP Ultra HD) @ 15fps

- Video Format: H.264 / H.265

- Field of View: Horizontal 108.4°–38° | Vertical 57.3°–21.2°

- Lens: f=2.8mm@F=1.6, f=8mm@F=1.6

- Day & Night: IR-cut filter with auto-switching

- Infrared Night Vision: Up to 30m (100ft), 850nm IR LEDs

- Colour Night Vision: 1 spotlight (6500K / 450 lumens)

- Audio: Two-way audio with built-in microphone & speaker

- Zoom Mode: 6× hybrid zoom

- Frame Rate:

Mainstream: 10fps – 15fps

Substream: 10fps – 15fps

- Bitrate:

Mainstream: 1024Kbps – 4096Kbps

Substream: 64Kbps – 672Kbps

- Pan Range: 355°

- Tilt Range: 0° ~ 90°

- Pan/Tilt Speed: Adjustable

- Presets: 32 positions

- Auto Tracking: Supported

- DC Power: 5V⎓2A (USB Type-C input)

- Battery: 7.2V 5100mAh rechargeable (36.72Wh, varies by settings & usage)

- Solar Power: Compatible with Reolink Solar Panel 2 for continuous charging

- Reset Button: Yes

- MicroSD Card Slot: Supports up to 512GB

- Audio I/O: Built-in microphone & speaker

- Network Connectivity: 4G LTE / 3G WCDMA (no WiFi required)

Supported Bands:

- EU Version (EC25-EU): LTE-FDD B1/B3/B7/B8/B20/B28A; WCDMA B1/B8

- US Version (EC25-US): LTE-FDD B2/B4/B5/B12/B13/B14/B66/B71

- AU Version (EC25-AU): LTE-FDD B1/B3/B5/B7/B8/B28; WCDMA B1/B5/B8

- Transmission Speed: LTE FDD: Up to 10Mbps (DL) / 5Mbps (UL); LTE TDD: Up to 8.96Mbps (DL) / 3.1Mbps (UL); HSPA+: Up to 42Mbps (DL) / 5.76Mbps (UL)

- Protocols & Standards: SSL, TCP/IP, UDP, UPnP, SMTP, NTP, P2P

- Operating Systems: Windows, macOS, iOS, Android

- User Access: Supports up to 12 simultaneous video streams (2 mainstream + 10 sub streams)

- Smart Detection & Alarms: PIR, person, vehicle, and pet detection

- PIR Detecting Angle: 100° horizontal

- Recording Modes: Motion-triggered recording

- Smart Alarm Options: Custom notifications and alerts

- Smart Home Integration: Works with Google Assistant & Reolink Cloud

- Weatherproof Rating: IP65

- Operating Temperature: –10°C ~ +55°C (14°F ~ 131°F)

- Operating Humidity: 10% – 90%

- Size: 228 × 147 × 129 mm

- Image Sensor: 1/3″ + 1/2.8″ CMOS Sensor

- Video Resolution: Up to 3840 × 2160 (8MP Ultra HD) @ 25fps

- Video Format: H.264 / H.265

- Field of View: Horizontal 104°–38° | Vertical 60°–21°

- Lens: f=2.8mm@F=1.6, f=8mm@F=1.6

- Day & Night: IR-cut filter with auto-switching

- Infrared Night Vision: Up to 30m (100ft), 2pcs/850nm IR LEDs

- Colour Night Vision: 2 spotlights (6500K / 450 lumens)

- Audio: Two-way audio with built-in microphone & speaker

- Zoom Mode: 6× hybrid zoom

- Mainstream: 2fps – 25fps (default configurable)

- Substream: 4fps – 20fps

- Mainstream Bitrate: 4096Kbps – 8192Kbps

- Substream Bitrate: 128Kbps – 1228Kbps

- Pan Range: 355°

- Tilt Range: 0° ~ 90°

- Pan Speed: 2.5° – 90°/s (adjustable)

- Tilt Speed: 1.5° – 60°/s (adjustable)

- Presets: 32 positions

- Tracking Method: Digital Tracking, Pan/Tilt Tracking First, Digital Tracking First

- Auto Tracking: Supported

- DC Power: 12V⎓2A, <12W

- Power Port: DC input

- Reset Button: Yes

- Ethernet: One 10M/100Mbps RJ45 port

- MicroSD Card Slot: Supports up to 256GB

- Audio I/O: Built-in microphone & speaker

- Wi-Fi Standard: IEEE 802.11 a/b/g/n

- Operating Frequency: Dual-band 2.4GHz & 5GHz

- Wireless Security: WPA-PSK / WPA2-PSK

- Web Browsers Supported: Chrome, Edge, Firefox, Safari

- Operating Systems: Windows, Mac OS, iOS, Android

- Protocols & Standards: HTTP, HTTPS, SSL, TCP/IP, UDP, UPnP, RTSP, RTMP, SMTP, NTP, DHCP, DNS, DDNS, P2P

- User Access: Supports up to 20 users (1 admin + 19 users), max 12 simultaneous streams (2 mainstream + 10 sub streams)

- Smart Detection & Alarms: Motion, person, vehicle, and pet detection

- Recording Modes: Motion-triggered (default), scheduled recording, continuous recording

- Smart Alarm Options: Custom notifications and alerts

- Smart Home Integration: Works with Google Assistant

- Weatherproof Rating: IP65

- Operating Temperature: -10°C ~ +55°C (14°F ~ 131°F)

- Operating Humidity: 10% – 90%

- Size: 228 × 147 × 129 mm